Review for Chapter 26 Quiz

1.  How much charge flows from a 12-V battery when it is connected to a 15.6 μF capacitor?  Answer – 1.87 x 10-4C

2.  A 0.40 μF capacitor is desired.  What area must the plates have if they are to be separated by a 4.0 mm air gap?  Answer – 180 m2

3.  A 3.00 μF and a 4.00 μF capacitor are connected in series and this combination is connected in parallel with a 2.00 μF capacitor.  (a)  What is the equivalent capacitance of this circuit?  (b)  If 26.0 V is applied across the whole circuit, calculate the voltage across each capacitor.   Answer – (a) 3.71 μF (b) 26.0V, 14.9 V, 11.1 V

4.  If 1200 V is applied to a 2800 pF capacitor, how much electric energy is stored?

Answer – 2.0 x 10-3J

5.  What is the capacitance of a pair of circular plates with a radius of 5.0 cm separated by 3.2 mm of mica (K = 7)?   Answer – 1.5 x 10-10F